Denso Parametric Test System – ECU Output, Load Current Measurement

This requirements document relates to FoDM Digital and PWM outputs that have Ball Systems Current Measurement boards attached for current measuring purposes.

Theory of Operation:  A single Analog Input channel of a DAQ board acquires a specified number of samples, determined by supplying the desired sample rate and length of the acquisition time.  The acquired waveform is averaged and a single, scaled Current value is returned back to the NI VeriStand system.

Analog Input, Current Measurement Custom Device behavior:

- Create a Trigger User Channel

- The Trigger User Channel is used to trigger an event in the test system, such as sending a CAN message and to trigger the start of an Analog Input waveform acquisition

- Only one Analog Input channel will be active at a time

- Inputs to the AI CM Acquisition and Analysis Custom Device

  - Trigger channel

  - AI Channel (on PXIe DAQ board)

  - CM Scaling:  0-5V CM output scaled to 0-X Amps of Current [Can be in Sys Def, based on spec of the Current Measurement board; Needs to be scaled on a per-channel basis since we have different CM boards with different current ranges.]

  - AI Sampling Rate [Can be in Sys Def]

  - Time delay between trigger and start of acquisition [Can be the same for all channels but should be settable during run-time.]

  - Acquisition Time [Defined as amount of time for acquiring samples to be averaged.  Can be the same for all channels but should be settable during run-time.  We can establish a “maximum acquisition time” in the Sys Def and then take a subset of the waveform to use in the average value evaluation, however.  Note:  If “Max Acq Time” value from Sys Def is used, that would then define the amount of time that test step would take to complete.  Slowest PWM is 100Hz and I would guess that 5 pulses would be the max we would want to average, so that helps us define what a “worst-case step time”would be.]

- Outputs from the AI CM Acquisition and Analysis Custom Device

  - Average Current (Amps)